9GRadio

Complete User Guide

RTL-SDR • All Supported Protocols

Version 1.1 (Illustrated) | 2026

Figure 0.1 — End-to-end signal chain: antenna → RTL-SDR → phone → DSP engine → outputs

# 1. Introduction

9GRadio is an Android SDR (Software-Defined Radio) application that turns any RTL-SDR USB dongle into a full-featured radio receiver. It supports analogue voice modes, digital voice protocols, data protocols, and aircraft/maritime tracking — all from one unified interface.

Figure 1.1 — How signal flows from antenna to the outputs you interact with in the app

## 1.1 Supported Hardware

- RTL-SDR Blog V4 (recommended — built-in HF upconverter, extended tuning)

- RTL-SDR Blog V3 and generic RTL2832U dongles (direct-sampling for HF)

- rtl_tcp / rtl_tcp_andro network sources

- Recorded IQ files (.raw, .cu8, etc.) for offline playback

## 1.2 Frequency Range

- Standard tuner range: 500 kHz – 1 766 MHz

- V4 / extended range: below 500 kHz and above 1 766 MHz (enable "Out of Band" in Settings tab)

Figure 1.2 — Approximate frequency coverage and where each mode/protocol sits

## 1.3 Quick-Start

- Plug your RTL-SDR dongle into the phone via OTG cable.

- Open 9GRadio — the app requests USB permission and connects automatically.

- Choose a mode from the Mode tab (e.g. NFM for VHF public-safety).

- Tune using the spectrum, waterfall, or Tune tab dial.

- Adjust Gain manually (start at 20–30 dB) with RTL AGC and Tuner AGC off.

# 2. Interface Overview

## 2.1 Main Screen

The main screen shows the spectrum (top), waterfall (middle), and the tabbed control drawer (bottom sheet). Tap and hold the frequency display to enter a frequency manually.

Figure 2.1 — Main screen: frequency display, spectrum, waterfall, and the Tune tab (tuning dial, step size, band presets)

## 2.2 Control Tabs

| Tab | Purpose |
| --- | --- |
| Mode | Select demodulation mode (analogue + digital) and memory slots |
| Tune | Tuning dial, step size, band presets, snap-to-channel |
| RF | Gain, AGC, squelch, volume, bias-tee, noise blanker/reducer, IF filter, sample rate, PPM |
| Display | FFT size, decimation (display zoom only — see §4.2), waterfall palette, spectrum theme, frame averaging, dB range |
| Rec | IQ and audio recording controls plus playback of saved files |
| Settings | Out-of-band mode, device info, scanner, bookmarks, EiBi, MLAT |
| APRS | Packet log, map (shown only in APRS mode) |
| ACARS | Message log (shown only in ACARS mode) |
| ADS-B | Aircraft table + live map/radar (shown only in ADS-B mode) |
| AIS | Ship table + live map (shown only in AIS mode) |
| DMR / YSF / D-STAR / Dig | Recent calls list (shown only in respective digital mode) |
| Scan | Frequency scanner (opens via Scanner button or FAB) |
| EiBi | Shortwave broadcast schedule browser (opens via EiBi chip on Mode tab) |

Figure 2.2 — Settings tab: Bookmarks, Storage Location, Scanner, MLAT, Device Info, and Out-of-Bound Frequency toggle

# 3. Supported Protocols & Modes

Every mode is selected from the Mode tab. Analogue modes appear on the top row; digital and data modes on the bottom row.

Figure 3.1 — Mode tab: analogue and digital mode buttons, plus the memory slot control and Shutdown Timer

## 3.1 Analogue Voice Modes

### AM — Amplitude Modulation

Used for shortwave broadcasts (3–30 MHz), aviation voice (108–137 MHz), and AM broadcast (530–1700 kHz).

| Setting | Recommended Value |
| --- | --- |
| Sample rate | 1.024 MS/s (HF); 1.92 MS/s (MW/SW) |
| Decimation | ÷64 → 16 kHz effective |
| Gain | Manual 20–35 dB, RTL AGC off, Tuner AGC off |
| IF / AM bandwidth | 4–6 kHz (SW), 9 kHz (MW channel spacing) |
| Squelch | −120 dB (open) for weak-signal SW |
| Direct sampling (HF) | Q-Branch (RTL-SDR V3/generic); Off for V4 |

★  For HF shortwave AM use 1.024 MS/s → Decimation ÷64 → 16 kHz. Higher sample rates cost battery without meaningful gain on most signals.

### NFM — Narrow FM

The default mode. Covers VHF/UHF public safety, amateur, marine, and commercial radio (typically 25 kHz channels).

| Setting | Recommended Value |
| --- | --- |
| Sample rate | 1.024 MS/s or 2.048 MS/s |
| IF bandwidth | 12.5–25 kHz (mode default auto) |
| Squelch | Start at −65 dB; raise until noise drops |
| Gain | 20–40 dB manual |

### WFM — Wide FM (Mono) / WFM ♫ — Wide FM Stereo

Standard FM broadcast (88–108 MHz). WFM ♫ enables stereo pilot decoding and RDS (Radio Data System) text display.

| Setting | Recommended Value |
| --- | --- |
| Sample rate | 1.92 MS/s or 2.048 MS/s |
| IF bandwidth | 200 kHz (auto) |
| Decimation | ÷1 (Off) — full bandwidth needed for pilot |
| RDS display | Appears automatically in WFM ♫ mode |

### USB / LSB — Single Sideband

USB (Upper Sideband) is the standard for HF ham, maritime, and aeronautical voice above 10 MHz. LSB is used below 10 MHz on ham bands.

| Setting | Recommended Value |
| --- | --- |
| Sample rate | 1.024 MS/s (HF) |
| IF bandwidth | 2.4–3.0 kHz (auto per mode) |
| Decimation | ÷64 → 16 kHz |
| Gain | 25–40 dB; adjust for minimum noise |
| Direct sampling | Q-Branch for V3/generic RTL-SDR on HF |

ℹ  USB and LSB always use automatic IF bandwidth — the slider is locked for these modes.

### CW / CWR — Morse Code

CW is standard Morse; CWR is reverse-polarity Morse. The IF filter can be narrowed from 800 Hz down to 100 Hz to improve S/N on weak signals.

| Setting | Recommended Value |
| --- | --- |
| IF bandwidth | 100–500 Hz (contest); 500–800 Hz (general) |
| Sample rate | 1.024 MS/s |
| Decimation | ÷64 |

### DSB — Double Sideband

Demodulates both sidebands simultaneously. Useful for AM signals without a carrier (DSB-SC), aircraft ILS, and some utility stations.

| Setting | Recommended Value |
| --- | --- |
| IF bandwidth | 4–6 kHz (default) |
| Use case | DSB-SC, ILS, utility HF |

## 3.2 Digital Voice Modes

All digital voice modes use the built-in mbelib-neo vocoder (statically linked — no external plugin needed). A "Recent Calls" tab appears automatically when a digital voice mode is selected.

### DMR — Digital Mobile Radio

TDMA digital voice used widely by public safety, commercial, and amateur radio. Supports Tier I (simplex), Tier II (conventional), and repeater operation.

| Setting | Recommended Value |
| --- | --- |
| Channel spacing | 12.5 kHz |
| Sample rate | 1.024 MS/s or 2.048 MS/s |
| Squelch | Typically −65 to −75 dB |
| Gain | Manual 20–40 dB |
| Vocoder | mbelib-neo built-in (no action required) |

★  The DMR tab shows caller ID, talk-group, time slot, and colour code from decoded frames.

### YSF — Yaesu System Fusion

Yaesu's digital voice mode used on the amateur VHF/UHF bands. Supports Voice Full Rate (VFR) and Voice Half Rate (VHR).

| Setting | Recommended Value |
| --- | --- |
| Channel spacing | 12.5 kHz |
| Sample rate | 1.024 MS/s |
| Squelch | −65 to −75 dB |

### D-STAR — Digital Smart Technologies for Amateur Radio

ICOM's GMSK digital voice protocol for amateur radio. Decoded frames show source/destination callsign, message, and GPS position when available.

| Setting | Recommended Value |
| --- | --- |
| Channel spacing | 6.25 kHz (DV) / 128 kHz (DD) |
| Sample rate | 1.024 MS/s |
| Squelch | −65 to −70 dB |

### DIG — Generic Digital (P25 / NXDN / other)

A catch-all digital mode that attempts to decode additional vocoder-based protocols not covered by the named modes above. Useful for experimentation.

### RAW — Raw IQ

Passes the raw complex IQ samples directly with no demodulation. Useful for inspecting signals, feeding external decoders, or verifying hardware.

## 3.3 Data & Telemetry Protocols

### APRS — Automatic Packet Reporting System

VHF packet-radio protocol (144.390 MHz in North America; 144.800 MHz in Europe) carrying position beacons, weather data, messages, and objects.

| Setting | Recommended Value |
| --- | --- |
| Fixed frequency | 144.390 MHz (NA) / 144.800 MHz (EU) — auto-set on mode select |
| Sample rate | 1.024 MS/s |
| IF bandwidth | Auto (locked) |
| Map | APRS tab shows station positions; tap a pin for details |
| Station timeout | 30 minutes (stations removed after silence) |

★  The APRS tab splits into a packet log (below) and a live map (above). Collapse the map to see more packets.

### ACARS — Aircraft Communications Addressing and Reporting System

VHF data link used by commercial aviation for pre-departure clearances, ATIS, and engine health reports. 9GRadio decodes all four ACARS frequencies simultaneously.

| Setting | Recommended Value |
| --- | --- |
| Centre frequency | 131.550 MHz (auto-set) — covers all four channels |
| Channels decoded | 131.550, 131.725, 131.825, 131.850 MHz (simultaneous) |
| Sample rate | ≥ 2.048 MS/s (auto-bumped on mode select) |
| Squelch | Leave open (−120 dB); ACARS decodes gated signals |

ℹ  ACARS mode forces a wide capture; sample rate is raised automatically if the current rate is insufficient.

### ADS-B — Automatic Dependent Surveillance–Broadcast

1090 MHz aircraft transponder data: position, altitude, speed, heading, ICAO hex code, and flight number. Decoded in real time on a live map with radar overlay.

| Setting | Recommended Value |
| --- | --- |
| Fixed frequency | 1090.000 MHz (auto-set) |
| Antenna | 1090 MHz vertical strongly recommended (spider, coax-stub) |
| Sample rate | 2.048 MS/s or 2.4 MS/s |
| Gain | 35–45 dB; reduce if strong signals cause intermod |
| Spectrum/waterfall | Can be disabled to save CPU (toggle in toolbar) |
| Aircraft timeout | 60 s without a new frame → removed from table/map |
| Record distance | All-time farthest aircraft persisted across sessions |
| MLAT | Multi-lateration supported via Settings > MLAT |

★  Long-press the Record Distance label to see full details of the best-ever aircraft catch.

### AIS — Automatic Identification System

VHF maritime transponder protocol. Decodes vessel name, MMSI, position, course, speed, and ship type on a live map.

| Setting | Recommended Value |
| --- | --- |
| Fixed frequency | 162 MHz (covers both AIS Ch 87B & 88B — auto-set) |
| Sample rate | 1.024 MS/s |
| Antenna | VHF marine whip or discone |
| Ship timeout | Configurable; default follows AIS_TARGET_TIMEOUT_MS |

# 4. RF & DSP Settings

## 4.1 Gain

Manual gain is strongly recommended for all analogue and digital modes. The gain slider sets the R82xx tuner LNA/Mixer/VGA stage.

| Control | When to use |
| --- | --- |
| Manual Gain (slider) | All modes — set once per band, then fine-tune for minimum noise |
| Tuner AGC | Only when you cannot set gain manually (moving vehicle, scanning wide range) |
| Hardware AGC | Rarely — controls the RTL2832U IF stage; leave off unless advised |

ℹ  RTL AGC and Tuner AGC should both be OFF for HF, SSB, CW, and weak-signal work.

## 4.2 Sample Rate & Decimation

The sample rate sets the captured IQ bandwidth. 9GRadio uses decimation in two distinct, independent ways — see the diagram below — and this section covers the automatic in-built decimation that feeds the demodulator, which provides real processing gain (≈3 dB per doubling of the decimation factor).

Figure 4.1 — Display decimation (visual only) vs. automatic in-built decimation (real processing gain)

Figure 4.1b — Display tab: the Decimation spinner (bottom right, currently Off) — this is the visual-only zoom control, not the DSP engine's processing-gain decimation

| Rate | ÷Decim | Effective BW | Ideal Gain | Best for |
| --- | --- | --- | --- | --- |
| 2.4 MS/s | ×150 | 16 kHz | +21.8 dB | Maximum SNR, HF |
| 2.4 MS/s | ×50 | 48 kHz | +17.0 dB | Wide HF coverage |
| 1.024 MS/s | ×64 | 16 kHz | +18.1 dB | Android (recommended) |
| 1.024 MS/s | ×32 | 32 kHz | +15.1 dB | NFM, APRS, AIS |
| 1.92 MS/s | ×1 | 1.92 MHz | +0 dB | WFM broadcast |
| 2.048 MS/s | ×1 | 2.048 MHz | +0 dB | ADS-B, ACARS |

Figure 4.2 — Processing gain achieved by common sample-rate / decimation combinations

★  For Android + HF: use 1.024 MS/s → ÷64 → 16 kHz. Move to 2.4 MS/s only when the ~3.7 dB gain is demonstrably useful and your phone can sustain it thermally.

## 4.3 IF Bandwidth (Protocol Filter)

The IF filter width limits how much of the spectrum the demodulator sees. Narrower = better SNR but more sensitive to frequency offset.

| Mode | Typical range | Notes |
| --- | --- | --- |
| CW / CWR | 100–800 Hz | Narrow for contest; wider for general CW |
| SSB (USB/LSB) | 2.4–3.0 kHz | Auto — slider locked |
| DSB | 4–6 kHz | Mode default auto |
| AM | 4–9 kHz | 5–6 kHz ideal for SW; 9 kHz for MW |
| NFM | 12.5–25 kHz | Auto per channel plan |
| WFM/WFM ♫ | 200 kHz | Auto — do not narrow |
| APRS | Auto | Locked — changing causes artefacts |
| DMR/YSF/DSTAR | 12.5 kHz | Fixed by protocol |
| ADS-B | Auto | Wideband; do not restrict |

## 4.4 Noise Blanker & Noise Reducer

- Noise Blanker — suppresses impulsive noise (ignition, switching power supplies). Enable on mobile installations.

- Noise Reducer — spectral subtraction post-demodulation. Reduces background hiss at the cost of some audio artefacts. Best on NFM voice.

ℹ  Both features reset their calibration when toggled — apply after the signal is stable.

Figure 4.4 — RF tab: Noise Processing, V4-specific features (Bias Tee, Direct Sampling), PPM, and Sample Rate selector

## 4.5 PPM Correction

Compensates for oscillator frequency error. Most dongles drift ±30–100 ppm. Use a known reference signal (e.g. a broadcast station or GPS-disciplined beacon) to calibrate. The value is shared between the RF tab slider and Settings > PPM.

## 4.6 Bias Tee

Supplies 5 V DC on the antenna connector to power an active antenna or LNA. Only available on RTL-SDR Blog V3/V4 and compatible hardware. Do NOT enable with a passive antenna — it will damage the dongle.

## 4.7 Direct Sampling (HF)

Allows HF signals to bypass the R82xx tuner and connect directly to the RTL2832U ADC. Required for HF reception on V3/generic dongles.

Figure 4.3 — HF signal path: direct sampling (V3/generic) vs. built-in upconverter (V4)

| Setting | Use |
| --- | --- |
| Off (Tuner) | All frequencies above ~24 MHz; V4 for all HF |
| Q-Branch | HF on V3/generic — recommended; lower noise than I-Branch |
| I-Branch | Experimental HF on V3/generic; try if Q-Branch is noisier |

# 5. Memory Slots & Bookmarks

## 5.1 Per-Protocol Memory Slots

Every demodulation mode has 9 independently cycling memory slots (Slot 0–8). Slot 0 is the mode's default; slots 1–8 let you store different frequency/gain/filter configurations for the same protocol.

- Tap [ Slot # ] to cycle forward (or backward with the reverse button).

- Each cycle saves the current settings into the outgoing slot before loading the new one.

- A never-used slot is automatically seeded from Slot 0 on first visit.

- Slot 9 is reserved for EiBi station tuning and is never cycled normally.

Figure 5.1 — Memory slot cycling order and the special-purpose Slot 9

ℹ  Settings saved per slot include: frequency, gain, AGC state, sample rate, decimation, IF bandwidth, squelch, volume, bias-tee, noise blanker/reducer, PPM, waterfall palette, spectrum theme, dB range, and more.

## 5.2 Bookmarks

Bookmarks store a frequency, mode, label, colour, and optional notes. They are organised into named lists and support favouriting. Access them via the Bookmark FAB or Settings tab.

## 5.3 Memory Channels

Memory channels are richer than bookmarks — they also store gain, squelch, bias-tee, direct-sampling mode, and PPM. Load a channel and the radio reconfigures completely. Save the current state as a channel from the Memory Activity.

# 6. EiBi Shortwave Schedule Browser

The EiBi tab is a built-in shortwave broadcast schedule (sourced from Eibi.de). It is not a demodulation mode — it overlays on whatever mode is active, letting you browse and tune stations without changing protocol.

## 6.1 Opening EiBi

- Tap the EiBi chip on the right end of the Mode tab's analogue row.

- The EiBi tab appears in the drawer. Tap any row to tune the radio to that station's frequency in the appropriate mode (AM/USB/LSB).

## 6.2 Filter Options

| Filter | Description |
| --- | --- |
| Language | Show only entries matching selected language codes |
| Mode | Limit to AM / USB / LSB / CW / DSB transmissions |
| Target Area | Filter by ITU target region (e.g. Eu, NAm, SEA) |
| Broadcast Only | Hide utility / non-broadcast stations |
| Reliable Only | Hide entries flagged irr/tent/test/alt/harm/imod in EiBi |
| On Air Now | Show only entries currently within their UTC transmission window |
| Hide Dead Bands | MUF-based propagation estimate: hides frequencies unlikely to propagate to your location right now |

★  "Hide Dead Bands" uses your GPS location and a compact F2-layer ionospheric model. It needs a location fix to work — no fix = no filtering.

## 6.3 Schedule Updates

The EiBi schedule auto-downloads when the tab is opened and silently refreshes every 4 hours while the tab is visible. Schedules older than 7 days are automatically re-fetched. EiBi publishes new schedules twice a year (ITU A/B seasons).

# 7. Frequency Scanner

The scanner is independent of demodulation mode. Open it via the Scanner FAB or the Scanner button on the Settings tab.

## 7.1 Scan Modes

- Scan — steps through a defined frequency range, stopping on active channels above the squelch threshold.

- Search — wider, continuous sweep optimised for discovering unknown activity.

## 7.2 Key Scanner Settings

| Setting | Description |
| --- | --- |
| Start / Stop frequency | Defines the sweep range |
| Step size | Frequency increment per hop |
| Squelch threshold | Signals below this are skipped |
| Dwell time | How long to listen on an active channel before moving on |
| Mode | Demodulation mode used during scan pauses |

ℹ  The scanner captures a pre-scan RF snapshot and restores it when scanning stops, so your regular mode settings are never overwritten.

# 8. Recording & Playback

## 8.1 IQ Recording

Records the raw complex I/Q samples as received from the dongle. The file can be replayed through any compatible SDR application (9GRadio, GNU Radio, SDR#, etc.) at the original sample rate.

- Start/stop from the Rec tab or the Record IQ button.

- Files are saved to the configured storage location (Settings > Storage).

- Format: unsigned 8-bit (u8) IQ, matching RTL-SDR conventions.

Figure 8.1 — Rec tab: Record IQ / Record Audio controls and a saved IQ recording (frequency, mode, sample rate, timestamp, file size)

## 8.2 Audio Recording

Records the post-demodulation audio as a WAV file at 48 kHz. The file can be played in any audio player.

## 8.3 IQ File Playback

Play back a previously recorded IQ file through the same DSP pipeline as a live device. Useful for offline analysis or demonstrating reception to others.

- Open via the "Play IQ File..." button in the Rec tab.

- Specify the original sample rate and centre frequency if the file name does not encode them.

- Looping mode replays the file continuously.

## 8.4 Recordings List

The Rec tab shows all recordings with type, duration, file size, and timestamp. Tap the play button to replay; tap share to export; tap delete to remove.

# 9. MLAT — Multi-Lateration (ADS-B)

MLAT improves aircraft position accuracy by comparing the Time Difference of Arrival (TDOA) of ADS-B signals between multiple cooperating ground stations. 9GRadio includes both a client and server implementation.

## 9.1 Enabling MLAT

- Go to Settings > MLAT to configure the station and network.

- A live MLAT status bar appears in the toolbar when the engine is running, showing peer count, observation exchange, and last fix.

## 9.2 MLAT Status Indicators

Figure 9.1 — Toolbar status-light meanings

| Colour | Meaning |
| --- | --- |
| ● Grey | MLAT idle / not enough peers |
| ● Amber | Waiting for station requirement or recent rejection |
| ● Green | Station requirement met — fix possible |

# 10. Tips & Troubleshooting

## 10.1 No Audio / Silence

- Check the Volume slider in the RF tab (should be > 0%).

- Check Squelch — if set too high, valid signals are gated out. Lower to −120 dB to open completely.

- Ensure the correct mode is selected (e.g. USB vs LSB for HF voice).

- Verify the RTL-SDR is connected and the connection state is "Connected".

Figure 10.1 — Step-by-step flow for diagnosing no-audio issues

## 10.2 USB Dropouts / Instability

- Try a shorter, higher-quality OTG cable.

- Reduce sample rate from 2.4 MS/s to 1.024 MS/s.

- Enable USB power saving workaround in the phone's developer options if available.

- Close other apps consuming CPU/USB bandwidth.

## 10.3 HF Reception Is Poor

- Use direct sampling Q-Branch for RTL-SDR V3/generic. The V4 has a built-in upconverter — leave direct sampling Off.

- Antenna quality matters more than SDR settings on HF. A long wire (10–20 m) is far more effective than doubling the sample rate.

- Disable Tuner AGC and RTL AGC; set gain manually to 20–35 dB.

- Use decimation ÷64 with 1.024 MS/s for maximum processing gain on narrow signals.

## 10.4 ADS-B Range Is Short

- Use a dedicated 1090 MHz antenna — a simple coaxial collinear or spider antenna outperforms a whip.

- Reduce gain if overload symptoms appear (broad noise peaks, ghosting). Try 35–40 dB.

- Disable spectrum/waterfall in the ADS-B tab toolbar to free CPU for decoding.

- Place the antenna outdoors or near a window with a clear sky view.

## 10.5 Digital Mode Not Decoding

- Confirm the frequency is correct — most digital modes have narrow channel plans.

- Ensure the squelch is low enough to pass the signal (−65 to −75 dB is a good starting point).

- For DMR/YSF/D-STAR: the mbelib-neo vocoder is always active; no plugin is needed.

- For ACARS: sample rate must be ≥ 2.048 MS/s — the app raises it automatically but verify after mode switch.

## 10.6 Battery / Thermal Management

- Use 1.024 MS/s instead of 2.4 MS/s for a 3.7 dB theoretical SNR trade-off that saves significant CPU, heat, and battery.

- Turn off the spectrum/waterfall display in ADS-B and AIS modes when not needed.

- Use the Shutdown Timer (Mode tab) to automatically disconnect the SDR after a set interval.

# 11. Decimation & SNR Reference

This section covers the automatic in-built decimation performed by the DSP engine (e.g. 2.4 MS/s down to 16 kHz), not the Display-tab decimation, which only zooms the FFT/waterfall and carries no processing gain (see §4.2).

Processing gain (SNR improvement from in-built decimation) follows: SNR = 10 × log₁₀(M) dB, where M is the decimation factor. Each doubling of M adds approximately 3 dB because the integrated noise bandwidth halves while the in-band signal is retained.

| Input rate | Factor (M) | Output BW | Processing gain | Best use |
| --- | --- | --- | --- | --- |
| 2.4 MS/s | 150 | 16 kHz | 21.8 dB | Max SNR, HF SW |
| 2.4 MS/s | 100 | 24 kHz | 20.0 dB | HF voice |
| 2.4 MS/s | 75 | 32 kHz | 18.8 dB | HF / NFM |
| 2.4 MS/s | 50 | 48 kHz | 17.0 dB | Wide HF |
| 1.024 MS/s | 64 | 16 kHz | 18.1 dB | Android recommended |
| 1.024 MS/s | 32 | 32 kHz | 15.1 dB | NFM, APRS |
| 1.024 MS/s | 16 | 64 kHz | 12.0 dB | Wide NFM |
| 1.024 MS/s | 1 | 1.024 MHz | 0 dB | Wideband scan |
| 2.048 MS/s | 1 | 2.048 MHz | 0 dB | ADS-B, ACARS |

ℹ  Starting at 2.4 MS/s provides a theoretical 3.7 dB processing-gain advantage over 1.024 MS/s for any given output rate, at the cost of higher USB bandwidth, CPU load, heat, and battery consumption.

9GRadio

Android SDR • RTL-SDR • All Protocols

AM  NFM  WFM  USB  LSB  CW  DSB  APRS  ACARS  ADS-B  AIS  DMR  YSF  D-STAR  DIG  RAW