9GRadio

Receiver Performance & Specifications Report

RTL-SDR-Compatible Software-Defined Radio Receiver — Android Application

| Application | 9GRadio (com.radiosport.ninegradio) |
| --- | --- |
| Version | 1.73 (versionCode 73) |
| Target hardware | RTL-SDR Blog V4 (R828D) / V4L “Lite” (R828S) — plus full support for RTL-SDR Blog V3 and generic RTL2832U + R820T/R820T2 dongles |
| Platform | Android 8.0+ (API 26–35), Kotlin + native C/C++ DSP |
| Report date | 22 September 2026 |
| Methodology | Static analysis of application source (driver, DSP, decoder code) |

Figures in this report are drawn directly from 9GRadio’s source code (USB/tuner driver register logic, DSP constants, and vendored decoder libraries) rather than from bench/RF measurement, and characterize the software-defined receiver chain built around the RTL2832U digitizer paired with an R828D, R828S, R820T, or R820T2 tuner.

# 1. Overview

9GRadio is a full-featured Android SDR receiver application for the RTL-SDR Blog V4 dongle family (RTL2832U digitizer paired with either an R828D or R828S tuner, 28.8 MHz TCXO reference), with full support for RTL-SDR Blog V3 and generic RTL2832U + R820T/R820T2 dongles as well. It re-implements the RTL-SDR USB driver stack, gain/tuning control, and DSP chain natively in Kotlin, and vendors a number of well-known open-source decoders (readsb, AIS-catcher, acarsdec, redsea, dsdcc, mbelib, and a custom Meteor-LRPT/SSTV chain) for protocol decoding.

The receiver accepts IQ from a local USB dongle, a networked rtl_tcp-compatible stream, or a recorded IQ file, and can simultaneously drive its own internal demodulator/decoder bank, stream discriminator audio to an external desktop decoder, and record raw IQ or demodulated audio — all from one tuned source.

This report characterizes the receiver as implemented in software (frequency coverage, gain/sample-rate tables, decode chain parameters, etc.). Figures such as noise figure and minimum detectable signal are the app’s own built-in calibration model (GainCalibration.kt) for driving its S-meter/AGC, not an independently bench-measured RF specification of the connected RTL-SDR hardware.

# 2. Platform & Build

| Property | Value |
| --- | --- |
| Package / application ID | com.radiosport.ninegradio |
| Version | 1.73 (versionCode 73) |
| Minimum OS | Android 8.0 (API 26) |
| Target / compile SDK | Android 15 (API 35) |
| Supported CPU ABIs | arm64-v8a, armeabi-v7a, x86_64 |
| Native build | CMake, C++17, -O3 |
| Implementation language | Kotlin (UI, control, DSP glue) + C/C++ (native DSP & vendored decoders via JNI) |
| Localization | 10 languages (en, fr, it, es, de, pt, ru, zh, ja + base) |

# 3. RF Front-End & Hardware

| Parameter | Specification |
| --- | --- |
| Digitizer / demodulator chip | RTL2832U |
| Tuner IC | R828D (V4) or R828S (V4L “Lite”) — auto-identified from EEPROM strings; R820T2 (or R820T) on RTL-SDR Blog V3 and generic RTL2832U dongles |
| Reference oscillator | 28.8 MHz TCXO (nominal 0 ppm drift) |
| Tunable frequency range | 500 kHz – 1,766 MHz, continuous, single front end |
| HF coverage method | V4 / V4L: built-in hardware upconverter, auto-engaged ≤ 28.8 MHz (no user action) V3 / generic (R820T/R820T2): no onboard upconverter — user must manually switch to Direct Sampling Mode (I- or Q-branch) via the RF tab for HF |
| RF band switching | 3-input (HF / VHF / UHF) with per-band notch filtering (V4/V4L) |
| VHF/UHF switch point | 250 MHz |
| Legacy 2-input switch point | 345 MHz (generic R828D / non-V4 dongles) |
| Bias tee | ≈ 4.5–5 V on antenna port, software-toggled (GPIO bit 3), one-tap or EEPROM force-on |
| Upconverter switch line | GPIO-5 (newer V4 hardware batches) |
| PPM (frequency) correction | ±50 ppm software LO compensation |
| USB identification | VID 0x0BDA; PIDs 0x2838 / 0x2832 / 0x2836 / 0x2837 / 0x2840 (plus vendor-specific clones); model resolved from EEPROM, not VID/PID |
| Host interface | USB OTG, bulk transfer |
| ADC resolution | 8-bit (native RTL2832U ADC) |

RTL-SDR Blog V4L (“Lite”, R828S) is distinguished purely by EEPROM manufacturer/product strings; register-level tuning logic is gated on detected tuner type rather than a separate driver path.

RTL-SDR Blog V3 and generic RTL2832U + R820T/R820T2 dongles are fully supported for tuning, gain control, sample-rate selection, and every demodulation/decode mode in this report. The one hardware-dependent difference is HF (below 28.8 MHz): V4/V4L auto-engage their built-in upconverter, while V3/generic dongles have none and require the user to manually enable Direct Sampling Mode (I- or Q-branch, RF tab) — the R820T/R820T2 tuner PLL cannot lock below ≈24 MHz otherwise.

# 4. Sample Rate & ADC

| Parameter | Specification |
| --- | --- |
| Sample-rate range | 240 kS/s – 2.5 MS/s |
| Number of presets | 27 discrete rates, in two non-contiguous operating bands |
| Low-rate band | 240,000 – 300,000 S/s (9 presets) — narrow-mode / low-CPU |
| Main band | 912,000 – 2,500,000 S/s (18 presets) — full-spectrum capability |
| Unusable “dead zone” | ≈ 300,001 – 899,999 S/s (demod resampler ratio aliases) |
| Rate-snapping behavior | Nearest-preset selection stays within the same band as the request (never crosses the dead zone) |

## 4.1 Low-Rate Band Presets (S/s)

| #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- |
| 240,000 | 242,000 | 250,000 | 256,000 | 264,000 | 272,000 |
| 286,000 | 288,000 | 300,000 |  |  |  |

## 4.2 Main Band Presets (S/s)

| #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- |
| 912,000 | 960,000 | 1,000,000 | 1,024,000 | 1,152,000 | 1,200,000 |
| 1,296,000 | 1,440,000 | 1,536,000 | 1,680,000 | 1,824,000 | 1,920,000 |
| 2,000,000 | 2,016,000 | 2,048,000 | 2,160,000 | 2,256,000 | 2,400,000 |
| 2,500,000 |  |  |  |  |  |

# 5. Gain Control

| Parameter | Specification |
| --- | --- |
| Gain architecture | 3-stage analog ladder — LNA / Mixer / VGA (R82xx tuner) |
| Manual gain steps | 29 discrete steps |
| Manual gain range | 0.0 dB – 49.6 dB |
| Automatic gain | Hardware AGC mode (alternative to manual table) |
| Saturation estimator | Monitors raw 8-bit IQ for samples ≤ 5 or ≥ 250 (ADC clipping proxy) |

## 5.1 Full 29-Step Manual Gain Table (index: dB)

| Steps 0–4 | Steps 5–9 | Steps 10–14 | Steps 15–19 | Steps 20–24/28 |
| --- | --- | --- | --- | --- |
| 0: 0.0 dB | 1: 0.9 dB | 2: 1.4 dB | 3: 2.7 dB | 4: 3.7 dB |
| 5: 7.7 dB | 6: 8.7 dB | 7: 12.5 dB | 8: 14.4 dB | 9: 15.7 dB |
| 10: 16.6 dB | 11: 19.7 dB | 12: 20.7 dB | 13: 22.9 dB | 14: 25.4 dB |
| 15: 28.0 dB | 16: 29.7 dB | 17: 32.8 dB | 18: 33.8 dB | 19: 36.4 dB |
| 20: 37.2 dB | 21: 38.6 dB | 22: 40.2 dB | 23: 42.1 dB | 24: 43.4 dB |
| 25: 43.9 dB | 26: 44.5 dB | 27: 48.0 dB | 28: 49.6 dB |  |

## 5.2 Built-In Scenario-Based Gain Recommendations

| Scenario | Suggested gain | Notes |
| --- | --- | --- |
| Strong local FM broadcast | Index 0 (0.0 dB) | Avoid ADC saturation |
| Broadcast FM (typical) | Index 3 (2.7 dB) |  |
| Weak-signal HF | Index 28 (49.6 dB, max) |  |
| VHF NFM, < 150 MHz | Index 12 (20.7 dB) |  |
| VHF/UHF NFM, 150–450 MHz | Index 10 (16.6 dB) |  |
| NFM, > 450 MHz | Index 14 (25.4 dB) |  |
| Satellite APT (NOAA/Meteor) | Index 22 (40.2 dB) |  |
| ADS-B (1090 MHz) | Index 16 (28.0 dB) | “Sweet spot” |
| APRS | Index 14 (25.4 dB) |  |
| Generic / search | Index 20 (37.2 dB) | Good all-round starting point |

# 6. Sensitivity & Noise Figure Model

9GRadio includes a built-in calibration model (GainCalibration.kt) used to convert raw ADC power to a calibrated dBm S-meter reading and to estimate minimum detectable signal (MDS). It is a software model for the UI/AGC — not an independent bench characterization — based on the RTL-SDR V4 datasheet and empirical reference figures.

| Frequency (MHz) | Reference noise figure (dB) |
| --- | --- |
| 30 | 8.5 |
| 100 | 5.2 |
| 200 | 4.8 |
| 433 | 5.5 |
| 868 | 6.8 |
| 915 | 7.0 |
| 1090 | 8.2 |
| 1296 | 9.5 |
| 1575 | 10.8 |
| 1700 | 12.0 |

## 6.1 Governing Formulas

- Thermal noise floor: kTB = −174 dBm/Hz (290 K reference)

- MDS ≈ kTB + 10·log₁₀(BW Hz) + NF(freq) + 6 dB (minimum usable SNR)

- ADC full-scale reference: ≈ −10 dBm

- Calibrated power (dBm) = 20·log₁₀(ADC power) + ADC full-scale − applied gain (dB) + NF(freq)

## 6.2 S-Meter Calibration (6 dB / S-unit, then +10 dB steps)

| S-unit | Level | S-unit | Level |
| --- | --- | --- | --- |
| S1 | −93 dBFS | S2 | −87 dBFS |
| S3 | −81 dBFS | S4 | −75 dBFS |
| S5 | −69 dBFS | S6 | −63 dBFS |
| S7 | −57 dBFS | S8 | −51 dBFS |
| S9 | −45 dBFS | S9+10 | −35 dBFS |
| S9+20 | −25 dBFS | S9+30 | −15 dBFS |
| S9+40 | −5 dBFS |  |  |

# 7. Analog Demodulation Modes

| Mode | Default bandwidth | Frequency range | Notes |
| --- | --- | --- | --- |
| AM | 10 kHz | 0.5 – 1,766 MHz | Amplitude modulation |
| FM | 200 kHz | 65 – 1,766 MHz | Wideband FM (general) |
| NFM | 12.5 kHz | 0.5 – 1,766 MHz | Narrowband FM (voice/PMR) |
| WFM | 200 kHz | 65 – 108 MHz | Broadcast FM, mono |
| WFM Stereo | 200 kHz | 65 – 108 MHz | Broadcast FM, pilot-tone stereo decode |
| USB | 3 kHz | 0.5 – 1,766 MHz | Upper sideband |
| LSB | 3 kHz | 0.5 – 1,766 MHz | Lower sideband |
| CW | 500 Hz | 0.5 – 1,766 MHz | Morse / continuous wave |
| CWR | 500 Hz | 0.5 – 1,766 MHz | CW, reversed sideband |
| DSB | 6 kHz | 0.5 – 1,766 MHz | Double sideband |
| RAW IQ | 200 kHz | 0.5 – 1,766 MHz | Undemodulated complex baseband |
| APRS | 12.5 kHz | 140 – 148 MHz | AFSK packet, incl. dual-watch mode ≥ 820 kS/s |

Internal IF processing is a two-stage integer decimation (device rate → mode-specific IF → 48 kHz audio), so no fractional resampling is used for standard-rate device sample rates.

A DemodMode.DRM enum entry exists in source (with a plausible bandwidth/frequency range and RF front-end plumbing) but has no corresponding entry in DemodulatorFactory.create() and no native decode library — it falls through to the raw IQ-magnitude demodulator with no OFDM/FEC/AAC decoding, and is not exposed in the main Mode-tab chip UI. Excluded from this report as non-functional.

# 8. Digital Voice Decoding

All digital voice modes share a common 12.5 kHz-class NFM discriminator pipeline. Frame synchronization / FEC is handled by a vendored dsdcc-derived decoder; voice vocoding (AMBE+2 / IMBE) is handled by vendored mbelib / mbelib-neo.

| Protocol | Channel BW | Frequency range | Standard / origin |
| --- | --- | --- | --- |
| DMR | 12.5 kHz | 136 – 1,766 MHz | ETSI TS 102 361-1 |
| P25 Phase 1 | 12.5 kHz | 136 – 900 MHz | TIA-102 |
| NXDN | 6.25 kHz | 136 – 512 MHz | ICOM / Kenwood |
| D-STAR | 12.5 kHz | 144 – 1,300 MHz | JARL, amateur |
| YSF (System Fusion) | 12.5 kHz | 144 – 450 MHz | Yaesu, amateur |
| M17 | 12.5 kHz | 144 – 1,000 MHz | M17 Project, amateur |
| dPMR | 6.25 kHz | 66 – 960 MHz | ETSI TS 102 658 |
| Dig (auto-detect) | 12.5 kHz | 136 – 1,766 MHz | Tries all known sync words; reports locked protocol |

- Each digital-voice tab groups consecutive frames into a single “Recent Calls” row (start time, duration, IDs, talker alias, flags, frame count) rather than one row per frame.

- Simultaneous external decode: raw pre-vocoder discriminator audio can be streamed live (16-bit PCM, 48 kHz default) to a desktop tool (DSD-FME, dsd-neo, SDRTrunk) over TCP without pausing the internal decoder.

# 9. Aircraft, Marine & Datalink Tracking

| Protocol | Frequency | Capability | Decode engine |
| --- | --- | --- | --- |
| ADS-B | 1090 MHz | Mode S / 1090ES, live list + radar scope + map (callsign, altitude, speed, position) | readsb (vendored, unmodified) |
| AIS | 161.975 / 162.025 MHz | Both marine VHF channels in one 300 kHz capture; live vessel map (MMSI, name, course, speed) | AIS-catcher (vendored) |
| ACARS | 129–131 MHz (8 presets) | Up to 4 channels monitored simultaneously; message log with registration/flight/label filters | acarsdec (vendored, custom JNI) |
| RDS | From any WFM station | PI code, Program Service name, RadioText; live overlay on spectrum display | redsea (vendored) |

## 9.1 ACARS Preset Channel List (MHz)

| Ch. 1 | Ch. 2 | Ch. 3 | Ch. 4 | Ch. 5 | Ch. 6 | Ch. 7 | Ch. 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 129.125 | 130.025 | 130.425 | 130.450 | 131.550 | 131.725 | 131.825 | 131.850 |

# 10. Image Decode Modes

| Mode | Format(s) / protocol | Tuning | Signal path |
| --- | --- | --- | --- |
| SSTV | Robot 36, PD120, Scottie, Martin, others (auto-detected from sync pulse) | Any tuned frequency | NFM discriminator audio (25 kHz default BW) |
| LRPT | Meteor-M2 weather satellite, 72 kBd QPSK, Viterbi + Reed–Solomon FEC | Any tuned frequency | Raw wideband IQ tap (120 kHz default BW), independent of narrow demod chain |

Both modes are “free-tune” (selectable at any frequency from the Mode tab) and are also auto-triggered by the satellite tracker when a tracked pass lands in the LRPT band (137–138 MHz) or on the ISS SSTV frequency (145.800 MHz).

# 11. Satellite Tracking & Doppler Correction

| Parameter | Specification |
| --- | --- |
| Orbit propagator | SGP4 (Vallado reference implementation, vendored unmodified) |
| Orbital elements (TLE) source | Celestrak amateur-satellite group; auto-refresh every 6 hours |
| Transponder database | SatNOGS DB (Libre Space Foundation), longer refresh cycle |
| Doppler correction update rate | ≈ 1 Hz (recalculated from real-time range rate + GPS position) |
| Prediction window options | 6 / 12 / 24 / 48 hours |
| Minimum elevation filter | 0° / 10° / 20° / 30° |
| Position source | Device GPS (automatic, no manual entry) |

# 12. Multilateration (MLAT) & Direction Finding (RDF)

## 12.1 Multilateration

| Parameter | Specification |
| --- | --- |
| Positioning method | Multilateration (TDOA) across ≥ 2 networked receiver stations |
| Protocol-agnostic ID keying | ADS-B (ICAO), AIS (MMSI), RDS (PI code), digital voice (radio ID) correlate the same transmission across stations |
| Best-effort analog support | AM/FM without decodable ID, keyed by carrier-onset timing on a shared frequency |
| Excluded | POCSAG/FLEX paging (fixed infrastructure — nothing to locate) |
| Upstream network compatibility | ADSBExchange, adsb.lol, private mlat-server (genuine upstream wire protocol) |
| Self-test | Rolling synthetic moving-target solve every cycle to verify timing/geometry/solver pipeline |
| Accuracy estimate | Live GDOP-based estimate at actual vs. ideal reference station spacing |

## 12.2 Direction Finding

| Parameter | Specification |
| --- | --- |
| Method | Live compass heading correlated against signal strength, plotted on a 360° ring |
| Hardware requirement | Directional antenna, rigidly mounted to the device |
| Targets | Any station lacking a position fix (ADS-B/AIS/ACARS/digital-voice “no pos”), or the live tuner frequency |
| Multi-station solve | Trilaterates using linked MLAT peers ranging the same target (single station yields a bearing only) |

# 13. Recording & File I/O

| Function | Format | Detail |
| --- | --- | --- |
| IQ recording — raw | .iq | Uncompressed 8-bit unsigned I/Q (native ADC format) |
| IQ recording — compressed | .iq.gz | Gzip-compressed 8-bit unsigned I/Q |
| IQ recording — float | .cf32 | 32-bit float complex I/Q |
| Demodulated audio recording | .wav | 16-bit PCM WAV |
| Live audio output | AudioTrack | 32-bit float PCM internal engine, 48 kHz default |
| Selectable audio sink rates | — | 48000 / 44100 / 32000 / 24000 / 22050 / 16000 Hz |
| External decoder audio stream | TCP, port 7355 default | Headerless 16-bit PCM, 48 kHz default (pre-vocoder discriminator audio) |

# 14. USB Streaming / Real-Time Pipeline

| Parameter | Specification |
| --- | --- |
| USB transfer model | Pipelined concurrent bulk transfers (SDR++ Android / rtl_tcp_andro pattern) |
| Concurrent transfer threads | 3 |
| Queue capacity | 12 buffers (4 × thread count) |
| Buffers pooled in flight | 16 total (3 active + 12 queued + 1), reused (no steady-state allocation) |
| Default transfer buffer size | 32 KB per transfer (≈ 512 KB pooled across all 16 buffers) |
| USB control timeout | 300 ms |

# 15. Spectrum / Waterfall Analysis

| Parameter | Specification |
| --- | --- |
| Default FFT size | 2,048 bins (fixed default across all modes) |
| Default frame averaging | 8x |
| Adaptive decimation (AM) | ÷16 — narrow AM channel |
| Adaptive decimation (NFM) | ÷32 — 12.5 kHz channel |
| Adaptive decimation (USB/LSB) | ÷32 — 3 kHz voice SSB |
| Adaptive decimation (CW/CWR) | ÷64 — single-tone, most aggressive |
| Adaptive decimation (APRS) | ÷16 — 12.5 kHz AFSK channel |
| Adaptive decimation (DMR/YSF/D-STAR) | ÷8 — timing-sensitive digital voice |
| Adaptive decimation (WFM/FM/RAW/ADS-B/etc.) | Off (1×) — full analysis bandwidth retained |

# 16. At-a-Glance Specification Summary

| Category | Headline figure |
| --- | --- |
| Tuning range | 500 kHz – 1,766 MHz |
| Sample rate | 240 kS/s – 2.5 MS/s (27 presets) |
| ADC resolution | 8-bit |
| Manual gain | 29 steps, 0.0–49.6 dB + hardware AGC |
| Frequency correction | ±50 ppm software PPM |
| Analog modes | 12 (AM/FM/NFM/WFM/WFM-St/USB/LSB/CW/CWR/DSB/RAW/APRS) |
| Digital voice protocols | 7 + auto-detect (DMR, P25p1, NXDN, D-STAR, YSF, M17, dPMR) |
| Datalink/tracking protocols | 4 (ADS-B, AIS, ACARS, RDS) |
| Image decode modes | 2 (SSTV, LRPT) |
| Memory slots | 9 per mode, independently saved/restored |
| IQ recording formats | 3 (.iq, .iq.gz, .cf32) |
| USB pipeline threads | 3 concurrent, 16 pooled buffers |
| Min. Android version | 8.0 (API 26) |

Report compiled from 9GRadio source v1.73 by static code analysis (USB/tuner driver, DSP constants, and decoder modules). Not an independently bench-verified RF test report.